SUBJECT: PHARMACEUTICAL ANALYSIS

PRACTICAL MANUAL BOOK

## LAB MANUAL PHARMACEUTICAL ANALYSIS – I

LIST OF EXPERIMENTS FOR I-B PHARM(2017-2018)– Projected PHARMACEUTICAL ANALYSIS I

| SL. No. | DATE | NAME OF THE EXPERIMENT | MARK | REMARKS |
| --- | --- | --- | --- | --- |
| STANDARDIZATION |  |  |  |  |
| 1 |  | SULPHURIC ACID |  |  |
| 2 |  | SODIUM HYDROXIDE |  |  |
| 3 |  | SODIUM THIOSULPHATE |  |  |
| 4 |  | POTASSIUM PERMANGNATE |  |  |
| 5 |  | CERRIC AMMONIUM SULPHATE |  |  |
| ASSAY |  |  |  |  |
| 6 |  | AMMONIUM CHLORIDE BY ACID BASE TITRATION |  |  |
| 7 |  | FERROUS SULPHATE BY CERIMETRY |  |  |
| 8 |  | COPPER SULPHATE BY IODOMETRY |  |  |
| 9 |  | CALCIUM GLUCONATE BY COMPLEXOMERY |  |  |
| 10 |  | HYDROGEN PEROXIDE BY PERMANGANOMETRY |  |  |
| 11 |  | SODIUM BENZOATE BY NON AQUEOUS TITRATION |  |  |
| 12 |  | SODIUM CHLORIDE BY PRECIPITATION TITRATION |  |  |
| ELECTROANALYTICAL METHODS |  |  |  |  |
| 13 |  | CONDUCTIMETRIC TITRATION OF STRONG ACID AGAINST STRONG BASE |  |  |
| 14 |  | POTENTIOMETRIC TITRATION OF STRONG ACID AGAINST STRONG BASE |  |  |
| 15 |  | LIMIT test for chlorides,sulphates & iron |  |  |

## Experiment No. 1	Date: ………………..

STANDARDISATION OF 0.1N H2SO4

AIM:

To determine the Normality of given sample of sulphuric acid.

## REFERENCE:

- Dr. G.Devalo Rao. Practical pharmaceutical inorganic chemistry

- Anees Ahamed Siddiqui and Mohammed Ali. Practical pharmaceutical chemistry.

## REQUIREMENTS:

Apparatus : conical flask, burette, pipette, standard flak

Reagents : H2SO4, sodium carbonate, methyl orange

## PRINCIPLE:

Standardization of sulphuric acid is done by acid base titration. When sulphuric acid is allowed to react with sodium carbonate, carbon dioxide and water is produced. In this titration, to locate the endpoint methyl orange indicator is used. Appearance of the pale yellow colour is the endpoint.

H2SO4 + Na 2CO 3	→   Na 2SO 4    + CO2    +	H2 O

## PROCEDURE:

Standardisation of 0.1 N H2SO4

Weigh accurately 0.15 g of anhydrous sodium carbonate previously heated at about 270oC for 1 hour. Dissolve in 100 ml water ,add methyl orange indicator, titrate until solution becomes faintly pink which persists for 30 seconds. Heat the solution to boiling cool and continue the titration until faint pink colour is no longer affected by heat.

Each ml of 0.1 M sulphuric acid is equivalent to 0.005299g of sodium carbonate

N 1V 1= N 2V 2

N 2 =   N 1V 1

V 2

REPORT:The strength of the given solution of H2SO4 is ……………………..

## Experiment No. 2	Date: ……………….

STANDARDISATION OF 0.1N NaOH

AIM:

To determine the Normality of given sample of sodium hydroxide.

## REFERENCE:

- Dr. G.Devalo Rao. Practical pharmaceutical inorganic chemistry.

- Anees Ahamed Siddiqui and Mohammed Ali. Practical pharmaceutical chemistry.

## REQUIREMENTS:

Apparatus : conical flask, burette, pipette, standard flask

Reagents : NaOH, Oxalic acid, phenolphthalein

## PRINCIPLE:

Standardization of sodium hydroxide is done by acid base titration. When oxalic acid is allowed to react with sodium hydroxide, sodium oxalate and water are obtained. In this titration, to locate the end point phenolphthalein indicator is used. Appearance of the pale pink colour which persists for 30 sec is the endpoint.

COOH	COONa

│	+ 2 NaOH	→	│	+ H2O COOH			COONa

## PROCEDURE:

Weigh accurately 0.63 g of oxalic acid into a 100 ml volumetric flask and make up to 100 ml volume with distilled water. Pipette out 20 ml solution and titrated 0.1 NaOH using phenolphthalein as indicator. Continue the titration to get the concordant value.

N 1V 1=N 2V 2

N 2 = N 1V 1

V 2

REPORT: The strength of the given solution of sodium hydroxide is………………………

## Experiment No. 3	Date:…………………….

STANDARDISATION OF 0.1N SODIUM THIOSULPHATE

AIM:

To determine the normality of given sample of sodium thiosulphate.

## REFERENCE:

- Dr. G.Devalo Rao. Practical pharmaceutical inorganic chemistry.

- Anees Ahamed Siddiqui and Mohammed Ali. Practical pharmaceutical chemistry.

.

## REQUIREMENTS:

Apparatus : Burette, Volumetric flask, Pipette,

Reagents : 0.1 N sodium thiosulphate, Potassium iodate, Potassium iodide.

## PRINCIPLE:

The principle of standardization of sodium thiosulphate is based on redox iodometric titration with potassium iodate as primary standard. Potassium iodate a strong oxidizing agent is treated with excess potassium iodide in acidic media which liberates iodine which is back titrated with sodium thioslphate. Uniformity of reactions between iodine and sodium thiosulphate forms basis for utilizing the standard solution of iodine in the analysis of sodium thiosulphate and use of sodium thiosulphate in the analysis of iodine.Starch indicator is used as indicator showing disappearance of blue colour

KIO3  + 5KI + 3H2SO4    → 3K2SO4	3I2	+ 3H2O I2    + 2Na2S2O3    →	2NaI + Na2S4O6

## PROCEDURE:

Preparation of 0.1N Sodium thiosulphate

Dissolve 24.8g of sodium thiosulphate pentahydrate(Na2S2O3.5H2O) in 800 ml of freshly boiled and cooled water and mix thoroughly by shaking for approximately 15 minutes. Make up the volume to 1000 ml.

## Preparation of 0.1N Potassium Iodate

Weigh accurately about 356 mg of KIO3 and dissolve in 100 ml distilled water

## Preparation of Starch indicator

Take 1 gm of soluble starch and triturate with 5 ml of water and add it to 100 ml of Boiling water containing 10 mg of Mercuric iodide with continous stirring

## Standardisation of 0.1N sodium thiosulphate

Take 10 ml of Potassium Iodate solution .Add 2 gm of Potassium Iodide and 5 ml of dilute H2SO4    ,keep it in dark for 10 minutes,add 2 to 3 drops of starch indicator and

titrate with sodium thiosulphte using starch solution as indicator until the blue colour is disappeared.

.

N 1V 1=N 2V 2

N 2 =   N 1V 1

V 2

REPORT: The strength of the given solution of sodium thiosulphate is ………….

## Experiment No: 4	Date: ……………….

STANDARDISATION OF 0.1 N POTASSIUM PERMANGANATE

AIM:

To determine the Normality of given sample of 0.1N Potassium permanganate.

## REFERENCE:

- Dr. G.Devalo Rao. Practical pharmaceutical inorganic chemistry.

- Anees Ahamed Siddiqui and Mohammed Ali. Practical pharmaceutical chemistry.

## REQUIREMENTS:

Apparatus : Burette, Iodine flask, Pipette,

Reagents : Potassium Permanganate and oxalic acid.

## PRINCIPLE:

The principle of standardization of potassium permanganate is based upon redox titration in which strength of an oxidizing agent is estimated by titrating it with a reducing agent and viceversa.Potassium permanganate acts as an strong oxidizing agent in acidic medium that oxidizers oxalic acid in to carbondioxide . Known strength of oxalic acid is titrated directly with potassium permanganate .End point can be detected with appearance of permanent pink colour potassium permanganate acts as self indicator

2KMnO4 +5H2C2O4 +3H2SO4 →K2SO4 +2MnSO4 +8H2O +10CO2

## PROCEDURE:

Preparation of 0.1 N Potassium permanganate solution

Dissolve 3.2g of potassium permanganate in 1000ml of water, heat on a water bath for 1 hour, allow to stand for 2 days. Filter the solution through glass wool.

## Preparation of 0.1 N Oxalic acid:

6.3 gm of oxalic acid dissolve in 1000 ml of distilled water

## Standardisation of 0.1N Potassium permanganate

Take 20 ml of Oxalic acid solution .add 5 ml of 1m sulphuric acid.warm the mixture to about 70oC .titrate with potassium permanganate solution taken in the burette. End point is appearance of pink colour that persist for 30 sec

N 1V 1=N 2V 2

N 2 = N 1V 1

V 2

REPORT:The strength of the given solution of potassium permanganate is………………

## Experiment No. 5	Date: ……………

STANDARDISATION OF 0.1 M CERIC AMMONIUM SULPHATE

AIM:

To determine the Normality of given sample of 0.1 M Ceric ammonium sulphate.

## REFERENCE:

- Dr. G.Devalo Rao. Practical pharmaceutical inorganic chemistry.

- Anees Ahamed Siddiqui and Mohammed Ali. Practical pharmaceutical chemistry.

## REQUIREMENTS:

Apparatus : Burette, Conical flask, Pipette,

Reagents : Ceric ammonium sulphate, Sulphuric acid, Arsenic trioxide, Sodium hydroxide, .Sulphuric acid

## PRINCIPLE:

Ceric ammonium sulphate is titrated	with Arsenic trioxide solution (Primary standard) in presence of sodium hydroxide, and osmic acid solution.

As 2O 3 + 2NaOH → 2NaAsO2 + H 2O

NaAsO2	+ 2H 2O → NaH2AsO4	+ 2H + +4e-

Ce 4+ + e- → Ce3+

## PROCEDURE:

Preparation of 0.1 M Ceric ammonium sulphate

Dissolve 65 g of ceric mmonium sulphate with the aid of gentle heat in a mixture of 30 ml of sulphuric acid and 500ml of water. Cool,filter dilute to 1000ml with water.

## Standardization 0f 0.1 M Ceric ammonium sulphate

Weigh accurately 0.2g of arsenic trioxide previously dried at 105 C for 1 hour and transferred to a 500ml conical flask. Wash the inner walls of the flask with 25 ml of 8

%w/v solution of sodium hydroxide dissolve and add 100 ml of water. Add 30 ml of dilute sulphuric acid , 0.15 ml of osmic acid, 0.1 ml of ferroin sulphate and titrate with ceric ammonium sulphate until the pink colour changes to pale blue

Each ml of ceric ammonium sulphate is equivalent to 0.004946 g of arsenic trioxide.

Molarity of cerric ammonium sulphate = W x RM

V x E

W=Weight of arsenic trioxide RM =Required molarity

E = Equivalent weight factor

V = Volume of cerric ammonium sulphate

REPORT: The strength of cerric ammonium sulphate is …………………

ASSAY

## Experiment No. 6	Date: ………………

ASSAY OF AMMONIUM CHLORIDE

AIM:

To determine the percentage purity of given sample of ammonium chloride.

## REFERENCE:

- Indian Pharmacopoeia -1985, volume I,

- Practical pharmaceutical inorganic chemistry, Dr. G.Devalo Rao

- Practical pharmaceutical chemistry by Anees Ahamed Siddiqui and Mohammed Ali

## CHEMICALS USED:

Ammonium chloride, NaOH, Oxalic acid, formaldehyde

## PRINCIPLE:

Ammonium chloride is assayed by indirect assay method. The sample is dissolved in water and treated with previously neutralised formaldehyde solution.(Methyleneimine is formed which polymerises to form Hexamine). This results in the quantitative liberation of HCl equivalent to NH4Cl. The liberated HCl is titrated with the standard solution of NaOH using phenolphthalein as indicator, appearance of pale pink colour is the indication of end point.

4NH4 Cl + 4H2O   4NH4OH + 4HCl

4 NH4OH + 6HCHO  (CH2)6N4 + 10 H2O

4HCl + 4NaOH  4NaCl + 4H2O

## PROCEDURE:

Standardisation of 0.1 NaOH

Weigh accurately 0.63 g of oxalic acid into a 100 ml volumetric flask and make up to 100 ml volume with distilled water. Pipette out 20 ml solution and titrated with 0.1 M NaOH using phenolphthalein as indicator. Continue the titration to get the concordant value.

## Assay of NH4Cl

Weigh 0.1gm of NH4Cl dissolve in 20 ml of H2O and add a mixture of 5 ml of previously neutralized formaldehyde solution and 20 ml water. After 2 minutes the contents of the conical flask is titrated against 0.1N NaOH using phenolphthalein as indicator. End point is the appearance of permanent pale pink colour.

Each ml of 0.1N NaOH is equivalent to 0.005349 gm of NH4Cl. Percentage purity can be determined by the following formula.

Titre value x Equivalent wt factor x Normality of NaOH(actual)

% =	 		 Weigh of sample x Normality of titrant (expected)

x 100

REPORT:	The percentage purity of the given sample of ammonium chloride is…………………

## ASSAY

Experiment No. 7	Date: …………………

ASSAY OF FERROUS SULPHATE

## AIM:

Determine the percentage purity of given sample of ferrous sulphate.

## REFERENCES:

1. Practical pharmaceutical inorganic chemistry by Dr. Deval Rao

## CHEMICALS USED:

Ferrous suphate, Cerric ammonium Sulphate, dil H2SO4.

## PRINCIPLE:

FeSO4 is an example of reducing agent and In the presence of dil H2SO4, FeSO4, is titrated with ceric ammonium sulphate using ferroin as indicator which gives a colour change from red to pale blue. Cerri ammonium sulphate is a powerful oxidizing agent and it is used only in acidic media because if the solution is neutral cerric hydroxide precipitates out. Cerric ion solution itself has intense yellow colour can be used as self indicator, but to increase the sensitivity of endpoint detection internal redox indicator like ferroin is added

## PROCEDURE:

Fe2+ →	Fe3+  + e- Ce4+  + e-    →	Ce3+

Fe 2+  + Ce4+ →	Fe 3+	+	Ce3+

## Preparation of 0.1 M Ceric ammonium sulphate

Dissolve 65 g of ceric mmonium sulphate with the aid of gentle heat in a mixture of 30 ml of sulphuric acid and 500ml of water. Cool,filter dilute to 1000ml with water.

## Standardization 0f 0.1 M Ceric ammonium sulphate

Weigh accurately 0.2g of arsenic trioxide previously dried at 105 C for 1 hour and transferred to a 500ml conical flask. Wash the inner walls of the flask with 25 ml of 8

%w/v solution of sodium hydroxide ,dissolve and add 100 ml of water. Add 30 ml of dilute sulphuric acid , 0.15 ml of osmic acid, 0.1 ml of ferroin sulphate and titrate with ceric ammonium sulphate until the pink colour changes to pale blue

Each ml of ceric ammonium sulphate is equivalent to 0.004946 g of arsenic trioxide.

Molarity of cerric ammonium sulphate = W x RM

V x E

W=Weight of arsenic trioxide RM =Required molarity

E = Equivalent weight factor

V = Volume of cerric ammonium sulphate

## Assay of FeSO4

Add 0.5 g of FeSO4 and 30 ml water .Add 20 ml of 1m sulphuric acid shake well, then titrate with 0.1N ceric ammonium sulphate solution using 0.1 ml ferroin sulphate as indicator until red colour disappears.

Each ml 0.1M Ceric ammonium sulphate is equivalent to 0.01519 gm of Ferrous sulphate.

Percentage purity can be determined by the following formula.

Titre value x Equivalent wt factor x Normality of titrant(actual)

% =	 		 Weigh of sample x Normality of titrant (expected)

REPORT: The percentage purity of Ferrous sulphate is……………….

x 100

## ASSAY

Experiment No. 8	Date: ………………..

ASSAY OF COPPER SULPHATE

## AIM:

Determine the percentage purity of given sample of copper sulphate sulphate.

## REFERENCES:

1.  Practical pharmaceutical inorganic chemistry by Dr. Deval Rao

## REQUIREMENTS

Apparatus: Iodine flask, Burette, conical flask

Reagents : Copper suphate, 0.1N Sodium thiosulphate solution, starch solution,

0.1 N KIO3 solution

## PRINCIPLE:

This is an iodometry type of titration. It depends upon the instability of cupric iodide which is formed in the reaction between CuSO4 and potassium iodide with the liberation of free iodine. When CuSO4 was allowed to react with KI in the presence of acetic acid, cupric iodide is formed.

2 CuSO4 + 4KI → 2CuI2 + 2K 2SO 4

CUPRIC IODIDE

The cupric iodide formed in the above reaction is unstable, so it decompose to give cuprous iodide with the liberation of free iodine.

2CuI2 → 2Cu 2I 2 + I2

CUPROUS IODIDE

The liberated iodine is titrated with 0.1N sodium thiosulphate using starch solution as indicator.

I2 + 2Na 2S 2O 3   → Na 2S 4O 6   + 2 NaI

Sodium tetrathionate

## PROCEDURE:

Preparation of 0.1N Sodium thiosulphate

Dissolve 24.8 g of sodium thiosulphate pentahydrate in 800 ml of boiled and cooledwater and mix thoroughly by shaking for approximately 15 minutes. Make up the volume to 1000 ml.

## Standardisation of 0.1N sodium thiosulphate

Take 10 ml of Potassium Iodate solution .Add 2 gm of Potassium Iodide and 5 ml of dilute H2SO4   ,keep it in dark for 10 minutes,add 2 to 3 drops of starch indicator and titrate with sodium thiosulphte using starch solution as indicator until the blue colour is disappeared.

.

N 1V 1=N 2V 2

N 2 =   N 1V 1

V 2

## Assay of copper sulphate

Weigh accurately about 1 gm and dissolve in 50 ml of water, add 3gm of KI, 5 ml of acetic acid and titrate the liberated iodine with 0.1n sodium thiosulphate using starch solution as indicator. Continue the titration till a faint blue colour remains, add 2g of potassium thiocyanate stir well and continue the titration until the blue colour disappears. Each ml of 0.1N sodium thiosulphate = 0.02497 gm of CuSO4

Percentage purity of the given Copper sulphate = VxE x AN x 100

W x RN

V =Volume of Na 2S 2O 3

E = Equivalent weight factor

AN= actual normality of Na 2S 2O 3

W = weight of sample

RN = Required Normality of Na 2S 2O 3

REPORT : The percentage purity of the given sample of copper sulphate is………………..

Experiment no.9	Date:………………..

# ASSAY OF CALCIUM GLUCONATE

## AIM:-

To perform the assay of given sample of calcium gluconate

## REFERENCE:-

I.p volume-I-1996 page no: 129.

## REQUIREMENTS:-

Apparatus : Conical flask, standard flask ,Burette,Pipette, Funnel

Reagents: Calcium gluconate,eriochrome black T indicator,Buffer,Disodium EDTA

## PRINCIPLE

Calcium salt is assayed by titration with 0.05m EDTA using ph buffer and eriochrome black T as indicator. EDTA forms calcium EDTA complex, and the color change of indicator, is from wine red to blue at the end point.This is based on Replacement titration.Magnesium forms complex with indicator

Mg2+ + indicator → Mg- ind

This magnesium indicator complex is more stable than Calcium indicator complex.Therefore Calcium has no effect with magnesium indicator complex.On titration with disodium edetate calcium edetate complex is formed

Ca2+   + EDTA	→ Ca-EDTA

When calcium is completely consumed ,next drop of EDTA breaks the mg-indicator complex,thus liberating the free indicator ,which gives the colour at the endpoint

## PROCEDURE

Standardization of 0.05 M EDTA

Take 10 ml of cacl2 ,add 5 ml of ammonia buffer (PH 10),2 drops of nindicator and titrate with EDTA(0.05 m)

## Assay of calcium gluconate

Weigh 0.5g calcium gluconate and dissolve it in 50ml of warm water. and add 5ml of 0.05m magnesium sulphate and 10ml of strong ammonia-ammonium chloride buffer solution ,titrate with 0.05m disodium EDTA using mordant black II as indicator .The endpoint is appearance of blue colour

Each ml of 0.05m disodium EDTA is equivalent to 0.02242g of calcium gluconate .

## REPORT

The percentage purity of calcium gluconate is……………………..

## Experiment : 10	Date……………………….

ASSAY OF HYDROGEN PEROXIDE

## AIM

To perform the assay of given sample of H2O2

## REQUIREMENTS :

Apparatus : conical flask , burette, pipette

Reagents : hydrogen peroxide, potassium permanganate, sulphuric acid

## PRINCIPLE

H2O2 is usually encountered in the form of an aqueous solution containing 6%,12%,30% volume and H2O2 is frequently is known as 20,40 and 100ml volume. This terminology is based on the volume of O2 librated when the solution is decomposed by boiling. The assay is done by volumetrically by oxidation reduction titration. When H2O2 is being titrated in an acidic medium KMnO4 is used as reducing agent.

2KMnO4 + 3H2SO4 + 5 H2O2 → K2SO4 + 2MnSO4 + 8H2O + 5O2

H2O2 is oxidized to give water and O2 2 H2O2	[O]	2 H2O + O2

In this titration KMnO4 act as a self indicator 68.04gm of H2O2 gives 22,400ml O2 1gm of H2O2 is equivalent to 329.2 ml of O2

## PROCEDURE

Preparation of 0.1 N Potassium permanganate solution

Dissolve 3.2g of potassium permanganate in 1000ml of water, heat on a water bath for 1 hour, allow to stand for 2 days. Filter the solution through glass wool.

## Standardisation of 0.1 M Potassium permanganate

To 25 ml of potassium permanganate solution in a glass stoppered flask, add 2g of potassium iodide, followed by 10 ml of sulphuric acid. Titrate the liberated iodine with 0.1M Sodium thiosulphate using 3 ml of starch solution , as an indicator.

N 1V 1=N 2V 2

N 2 =   N 1V 1

V 2

## REPORT:

Assay of hydrogen peroxide

To 1ml of H2O2 add 20 ml of 1M H2SO4.titrate against 0.1N KMnO4 Each ml of 0.02M KMnO4 is equivalent to 0.001701 gm of H2O2

The given sample of H2O2 solution contains…………………….

## Experiment no.11	Date:

ASSAY OF SODIUM BENZOATE BY NON-AQUEOUS TITRATION

## AIM:-

To find out the percent age purity of given sample sodium benzoate.

## REFERRENCE:-

1.	Pharmaceutical analysis. Anees A Siddiqui

## REQUIREMENTS

Apparatus: Conical flask, Burette

Reagents : Sodium benzoate, 0.1 M HClO4, Potassium hydrogen phthalate, Glacial acetic acid

## PRINCIPLE:-

sodium benzoate is the sodium salt of benzoic acid. It is widely used as chemical preservative in carbonated and still beverages, syrups, olives, sauces, relishes, jellies jams and pastry low fat salad dressing, fruit salads, prepared salads and in storage of vegetables.

It is a basic compound . On adding Glacial acetic acid and then titrating with strong acid perchloric acid ,this Glacial acetic acid acts as a base.

CH3COOH + HClO4 →	CH3COOH2+ +  ClO4-

## Assay of sodium benzoate

weigh 0.25g sodium benzoate and dissolved in 20ml Anhydrous glacial acetic acid in a 250 ml volumetric flask.mix it well warm to 500c. Add 2-3 drops of Crystal violet indicator. Titrate with 0.1M HClO4. While mixing the water and ether layer well by shaking until a light green color persists in the water layer.

Each ml of 0.1M HClO4 is equivalent to 0.01441g of sodium benzoate.

## REPORT:

The percentage purity of sodium benzoate is………….

## Experiment no.12	Date:…………

ASSAY OF SODIUM CHLORIDE BY PRECIPITATION TITRATION

## AIM:-

To find out the percent age purity of given sample sodium chloride.

## REFERRENCE:-

Practical pharmaceutical analysis. G.Devala rao

## REQUIREMENTS

Apparatus: Conical flask, Burette

Reagents : Sodium chloride, Potassium chromate.

## PRINCIPLE:-

sodium chloride is an example of electrolyte replenisher. It is assayed by Mohr’s method. The sample is dissolved in water and titrated against a standard solution of silver nitrate using potassium chromate as indicator. At the end point due to the complete precipitation of chloride as silver chloride, a slight excess of silver nitrate reacts with potassium chromate to give a red colored silver chromate.

NaCl	+	Ag NO3 →	NaNO3	+	AgCl K2CrO4 + 2AgNO3 →		Ag2CrO4 +		2 KNO3

This method is based on the fact that silver halide is more insoluble than silver chromate. Hence as long as there is any chloride left in the solution, no silver chromate is formed. Even though if formed, will immediately change to silver chloride according to the following equation

Ag2CrO4 + 2Cl- → 2AgCl + CrO 4-2

## Assay of sodium chloride

Weigh accurately 0.1g sodium chloride previously dried at 110 C for 2 hours and dissolve in 5 ml of water. Add 2 drops of potassium chromate as indicator and titrate with silver nitrate solution until a permanent reddish brown precipitate is formed. Repeat the titration for concordant values.

Each ml of 0.1M silver nitrate is equivalent to 0.005844 g of sodium chloride

## REPORT

The Percentage purity of sodium chloride is……………..

ELECTRO-ANALYTICAL METHODS

## Experiment :13	Date:……….

CONDUCTOMETRIC TITRATION OF STRONG ACID WITH STRONG BASE AIM

To perform conductometric titration of hydrochloric acid with sodium hydroxide and to determine the end point.

## REFERENCE

1.	Bhoomika R Gpoal. Hardik G Bhatt. Mayur M Patel. Pharmaceutical analysis –I. Page No: 99-100.

## REQUIREMENTS

Apparatus : Beaker, Pipette, Volumetric flask, conductivity cell

Reagents : 0.01 M Hydrochloric acid, 0.01 M Sodium hydroxide

## PRINCIPLE

The solution of HCl shows fairly significant conductivity mainly due to two species H+ and Cl-. For each amount of NaOH added, an equivalent amount of H+ ion is consumed according to the following reaction.

H+ + NaOH == Na+ + H2 O

The highly conducting H+ cations are effectively replaced by the relatively poorly conducting Na+ ions and consequently the conductance decreases. This continues until sufficient NaOH has been added to react with all the H+ ions present and the mixture contains only Na+ and Cl- ions in water

HCl + NaOH == H2 O + NaCl

This is called equivalence point. Further addition of NaOH simply augments the quantity of Na+ and OH- ion in the mixture and increase the conductance due to higher OH- anions. As a result, the conductomeric titration curve (conductance vs. volume of NaOH) has a minimum at the equivalence point. The position of the equivalence Point can be localized as the point of inter section of two straight lines, a descending segment before and an ascending segment after the minimum observed. Usually in this experiment, the concentration of solution in the burette should be approximately 10 times than that of the other solution, under these conditions the change in concentration due to dilution of the solution during the titration , will have minimum effect on the conductance measurements.

## PROCEDURE

Transfer 50 ml 0.01M hydrochloric acid solution in a 100 ml beaker. Immerse the conductivity cell in the solution in the beaker such that the tip of the electrode remains dipped in the solution. Measure the conductance for the pure HCl solution. Titrate with 0.1 M NaOH solution using conductometer, adding aliquot of 0.5ml of

0.1 M NaOH solution each time. Remember to mix well after each addition of titrant before measuring the conductance. Continue in this way until you have measured about six points beyond the endpoint. Plot the graph of volume of titrant against conductance.

## REPORT

From graph, endpoint of the conductometric titration is found to be	ml.

## Experiment : 14	Date :

POTENTIOMETRIC TITRATION OF A STRONG ACID WITH A STRONG BASE AIM

To perform the titration of 0.1 N HCl with 0.1 N NaOH by potentiometry and to locate the end point.

## REFERRENCE

1.	G.devala rao. Pharmaceutical analysis. Page No: 136

## REQUIREMENTS

Apparatus : pH meter, Magnetic stirrer, Beaker

Reagent : 0.1 N HCl, 0.1 N NaOH, Distilled water.

## PRINCIPLE

Potentiometric determination of the end point depends on the fact that the potential across the two electrodes (reference and indicator) immersed in the solution changes sharply at the equivalence or end [point. This change is similar to the colour change by an indicator in usual method. But the potentiometric method is more accurate. These titrations are useful when no suitable colour indicators are available. Equivalence point can be accurately found out after plotting normal plot (volume of titrrant vs. potential)

## PROCEDURE

In a 250 ml beaker add 10 ml of 0.1N HCl and add around 100 ml of waater of dip the electrodes properly. Keep the beaker on magnetic stirrer. Note the potential without adding any alkali slowly add with stirring, known volumes of 0.1 N NaOH solution, and note the potentials. enter the values in a tabular form. from the data (volume of titrant vs. potential). Plot the graph and calculate the end point.

## REPORT

The end pont in the titration of 0.1 N HCl with 0.1 N NaOH by potentiometry is